Version information:

Version REV2025

Date: 08/05/2025

MLKPAI-FMin01 Hardware User Manual WeChat Scan Code to register for Milian Technology Forum www.uisrc.com enjoy more resources for free

Scan the code to follow the WeChat public platform "MILIANKE" to learn more information

Technical support

Customers can obtain the customer service contact information of the official Milian Ke store through the following link, join the Milian Ke Enterprise WeChat technical support group, and get professional technical support and answers. Thank you for your support!

JD Milian Ke flagship store:https://milianke.jd.com Tmall Milian Ke flagship store:https://milianke.tmall.com

Taobao Milian Ke Ecological Store:https://milianke.taobao.com

After-sales

1: 7 days for no reason to return (except for human reasons).

2: Warranty period: Our products provide one-year warranty service from the date of express delivery (except for main chips, such as FPGAs or CPUs).

3: For repair and exchange, you need to provide the Taobao order number or contract number, and contact sales/technical support to arrange the return.

For after-sales maintenance, please log in to the work order system: https://www.uisrc.com/plugin.php?id=x7ree_service

4: The following situations are not covered by warranty

A: Damage to the board caused by improper use by users: such as short circuit of the development board caused by excessive voltage, solder pad falling off caused by self-soldering, peeling of copper wires, etc.

B: Improper daily maintenance of the user causes damage to the board: such as improper placement leading to corrosion of the circuit board, cracks in the substrate, etc

5: For products outside the scope of warranty (Article 4 above) and outside the warranty period, our company provides paid maintenance services. The maintenance only charges the cost of device materials, and the round-trip freight is all borne by the customer.

6: Return address, log in to the website to get the latest after-sales address:https://www.uisrc.com/t-1982.html

Changzhou Milianke Information Technology Co., Ltd. All rights reserved

Changzhou Liyang Headquarters: Building 10-1, Yachuang Hi-Tech Manufacturing Valley, Wutandu Road, Zhongguancun, Liyang City, Changzhou Nanjing R&D Base: Liandong U Valley, No. 3118, Jiyin Avenue, Moling Street, Jiangning District, Nanjing Nanjing Jiulong Lake Intelligent Manufacturing Innovation Port Building 3, 9th floor

| version | Time | description |
| --- | --- | --- |
| Rev2025 | 2025-08-05 | First edition |

directory

1 Overall overview.................................................................................................................................................................................  5 2 Overview of hardware parameters......................................................................................................................................................................... 5 3 Integrated board ....................................................................................................................................................................................  6

4 Hardware Detailed Description........................................................................................................................................................................ 7 1：SF1 SOC ....................................................................................................................................................................... 7 2：FLASH .........................................................................................................................................................................  7 3：UART ...........................................................................................................................................................................  8 4：JTAG ............................................................................................................................................................................  8

5：DVP .............................................................................................................................................................................. 9 6: Buzzer, keys, DIP switch and LED ............................................................................................................................ 11

7：HDMI .......................................................................................................................................................................... 12 8: TF card slot .......................................................................................................................................................................  13 9：EEPROM&RTC ......................................................................................................................................................... 13 10:25MHz crystal oscillator. ............................................................................................................................................................. 14

11: CEP interface ................................................................................................................................................................... 14 5 EG4S20NG88 IO Multiplexing Table.................................................................................................................................................... 16 6 Power Management..............................................................................................................................................................................  18

7 Naming conventions............................................................................................................................................................................... 19 1: Core module naming conventions.................................................................................................................................................... 19

2: Development platform naming rules.................................................................................................................................................... 19

User Notice - Must See

Core board users: Hello! Thank you for using our products, in order to avoid problems during the development process, please read the following points before use.

- The hardware manual explains the parameters of the development board from the schematic diagram of the development board, if you have any questions, please check the schematic or contact sales, technical support.

- Please use a regulated power supply for power, not a power supply with a large ripple.

- Please do not make changes to the components on the core board, and contact customer service or technical support if necessary.

- If you do not contact customer service or technical support, you will make changes to the development board, causing damage to the development board, which is not covered by the warranty and must be borne by the user.

Changzhou Milianke Information Technology Co., Ltd

Technical Service Department

# 1 Overall overview

The MLKPAI-FMin01 launched by Milianke will lead most of the user IO of Anlu Technology's EG4S20NG88 chip, and the development board is small and has good scalability, and the pin spacing of the development board is designed to be directly installed on the breadboard to facilitate users to expand the module by themselves. FMin01's JTAG download, UART communication and power supply are all integrated on TYPE-C, realizing that a TYPE-C line can achieve two functions of power supply and communication, which greatly saves space.

# 2 Overview of hardware parameters

| Overview of the hardware parameters of MLKPAI-FMin01 |  |
| --- | --- |
| Main chip model | EG4S20NG88 |
| Flash | W25Q16DVSN:16M-bit/2M-byte |
| UART&JTAG | Shared TYPE-C interface |
| DVP interface | Multiplexed with the derived user IO, it is compatible with the MLK-CAM004 camera module |
| Buzzer, buttons, DIP switch and LEDs | Passive buzzer, independent buttons x2, four-position DIP switch, LEDx8 |
| HDMI output | Support 720P/60FPS |
| TF card slot | Support TF card slot for data storage |
| EEPROM | 2K-bit |
| RTC | A backup power supply can be used via the pin header that leads out |
| Crystal vibration | 25MHz on a single end |
| CEP | There are a total of 40 user IOs that lead to users on both channels |
| Power supply | USB 5V power supply |
| Dimensions | 26x62mm, can be plugged directly into the breadboard |

# 3 Integrated board

Front:

Reverse side:

# 4 Hardware Detailed Description

### 1：SF1SOC

| EG4S20NG88 chip parameters |  |
| --- | --- |
| LUTs | 19600 |
| FFs | 19600 |
| Dis-Ram bits | 156800 |
| ERAM(9K) | 64 |
| ERAM(32K) | 16 |
| M18x18 | 29 |
| SRAM | 4988928 |
| PLL | 4 |
| Low-skew GCLK | 16 |
| EM SDRAM | 2Mx32bits |
| User IO Banks | 8 |
| Maximum User IOs | 71 |

MLKPAI-FMin01 is equipped with an Anlu programmable SOC chip EG4S20NG88. The chip integrates 2Mx32bit SDRAM (EM638325), with a maximum operating frequency of 200Mhz and a maximum read and write bandwidth of up to 800MB/s.

## 2：FLASH

SPI FLASH, model W25Q16DVSN, can be used to save data and code. Main technical parameters: 16Mbit

•Standard SPIs: CLK, /CS, DI, DO, /WP, /Hold

•Clock rate: 104MHz

•Operating voltage: 3.3V

FLASH Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |  | description |
| --- | --- | --- | --- | --- |
| AS_rCSO | 69 | LVCOMS33 | Film selection |  |
| AS_DATA | 80 | LVCOMS33 | The main collection and issuance |  |
| AS_DCLK | 82 | LVCOMS33 | clock |  |
| AS_ASDO | 81 | LVCOMS33 | The main issue is followed by collection |  |

## 3：UART

In order to integrate with JTAG on the same TYPE-C interface, MLKPAI-FMin01 is equipped with a USB hub chip, and at the same time, a USB conversion chip is used to realize the FPGA to communicate with the outside world through the serial port.

UART Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD | description |
| --- | --- | --- | --- |
| FPGA_UART_TXD | 17 | LVCOMS33 | FPGA UART serial port send |
| FPGA_UART_RXD | 16 | LVCOMS33 | FPGA UART serial port reception |

## 4：JTAG

In order to integrate with UART on the same TYPE-C interface, MLKPAI-FMin01 integrates the Anlu low-speed downloader solution, which can be compiled and debugged by the compiler launched by Anlu through the data cable, and the TYPE-C data cable is also the total power supply for power supply, saving cables.

JTAG Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |  | description |
| --- | --- | --- | --- | --- |
| JTAG_TMS | 22 | LVCOMS33 | JTAG interface |  |
| JTAG_TCK | 26 | LVCOMS33 | JTAG interface |  |
| JTAG_TDO | 21 | LVCOMS33 | JTAG interface |  |
| JTAG_TDI | 25 | LVCOMS33 | JTAG interface |  |

## 5：DVP

The MLKPAI-FMin01 leads the DVP interface and can be adapted to the MLK-CAM004 camera module. The DVP interface leads the signal line to be fully multiplexed with the J4 headers.

DVP Pin Assignment Table:

| DVP PIN |  | FPGA PIN | IOSTANDARD |
| --- | --- | --- | --- |
| 1 | VCC_3V3 | \ | \ |
| 2 | VCC_3V3 | \ | \ |
| 3 | B4_BE2N_40 | 40 | LVCOMS33 |

| 4 | B4_BE2P_41 | 41 | LVCOMS33 |
| --- | --- | --- | --- |
| 5 | B4_BE1P_37 | 37 | LVCOMS33 |
| 6 | B4_BE1N_38 | 38 | LVCOMS33 |
| 7 | GND | \ | \ |
| 8 | \ | \ | \ |
| 9 | \ | \ | \ |
| 10 | GND | \ | \ |
| 11 | \ | \ | \ |
| 12 | \ | \ | \ |
| 13 | B3_BE2N_33 | 33 | LVCOMS33 |
| 14 | B3_BE2P_32 | 32 | LVCOMS33 |
| 15 | GND | \ | \ |
| 16 | \ | \ | \ |
| 17 | \ | \ | \ |
| 18 | B3_BE1N_28 | 28 | LVCOMS33 |
| 19 | B3_BE1P_29 | 29 | LVCOMS33 |
| 20 | GND | \ | \ |
| 21 | B1_L2P_10 | 10 | LVCOMS33 |
| 22 | B1_L2N_11 | 11 | LVCOMS33 |
| 23 | B1_L3P_12 | 12 | LVCOMS33 |
| 24 | B1_L3N_13 | 13 | LVCOMS33 |
| 25 | GND | \ |  |
| 26 | B8_TE1N_87 | 87 | LVCOMS33 |
| 27 | B8_TE1P_86 | 86 | LVCOMS33 |
| 28 | B7_TE1P_70 | 70 | LVCOMS33 |
| 29 | B7_TE1N_71 | 71 | LVCOMS33 |
| 30 | GND | \ | \ |
| 31 | \ | \ | \ |
| 32 | \ | \ | \ |
| 33 | B6_R2P_64 | 64 | LVCOMS33 |
| 34 | B6_R2N_63 | 63 | LVCOMS33 |
| 35 | \ | \ | \ |
| 36 | \ | \ | \ |
| 37 | B1_L1P_5 | 5 | LVCOMS33 |
| 38 | B1_L1N_4 | 4 | LVCOMS33 |
| 39 | \ | \ | \ |
| 40 | \ | \ | \ |

## 6: Buzzer, buttons, dip switch and LED

The MLKPAI-FMin01 integrates 8 LED red lights, a passive buzzer, a four-position DIP switch, and two buttons.

LED Pin Distribution Table:

| Signal name | FPGA PIN | IOSTANDARD |
| --- | --- | --- |
| B1_L4_14 | 14 | LVCOMS33 |
| B1_L2_3 | 3 | LVCOMS33 |
| B1_L1_2 | 2 | LVCOMS33 |
| B5_R2_51 | 51 | LVCOMS33 |
| B5_R3_52 | 52 | LVCOMS33 |
| B6_R1_57 | 57 | LVCOMS33 |
| B6_R2_59 | 59 | LVCOMS33 |
| B6_R3_62 | 62 | LVCOMS33 |

Passive Buzzer Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |
| --- | --- | --- |
| B3_B4_27 | 27 | LVCOMS33 |

DIP Switch Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |
| --- | --- | --- |
| B5_R1_45 | 45 | LVCOMS33 |
| B3_B1_23 | 23 | LVCOMS33 |
| B2_L2_19 | 19 | LVCOMS33 |
| B2_L1_18 | 18 | LVCOMS33 |

Key Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |
| --- | --- | --- |
| B3_B6_31 | 31 | LVCOMS33 |
| B3_B5_30 | 30 | LVCOMS33 |

## 7：HDMI

MLKPAI-FMin01 supports HDMI 720P/60FPS output.

HDMI Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD | description |
| --- | --- | --- | --- |
| B6_R1P_61 | 61 | LVCOMS33 | HDMI data differential pair P-side |
| B6_R1N_60 | 60 | LVCOMS33 | HDMI data differential pair N-end |
| B5_R2P_49 | 49 | LVCOMS33 | HDMI data differential pair P-side |
| B5_R2N_50 | 50 | LVCOMS33 | HDMI data differential pair N-end |
| B5_R1P_47 | 47 | LVCOMS33 | HDMI data differential pair P-side |
| B5_R1N_48 | 48 | LVCOMS33 | HDMI data differential pair N-end |
| B5_R3P_55 | 55 | LVCOMS33 | HDMI clock differential pair P-end |
| B5_R3N_54 | 54 | LVCOMS33 | HDMI clock differential pair N-end |

## 8: TF card slot

The MLKPAI-FMin01 is equipped with a TF card slot for data storage.

TF Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |  | description |
| --- | --- | --- | --- | --- |
| B7_T5_74 | 74 | LVCOMS33 | TF card data 2 |  |
| B7_T6_75 | 75 | LVCOMS33 | TF card data 3 |  |
| B7_T7_76 | 76 | LVCOMS33 | TF card command |  |
| B7_T2P_78 | 78 | LVCOMS33 | TF card clock |  |
| B7_TE2N_77 | 77 | LVCOMS33 | TF card data 0 |  |
| B7_T8_79 | 79 | LVCOMS33 | TF card data 1 |  |

## 9: EEPROM & RTC

The MLKPAI-FMin01 is equipped with an EEPROM with a capacity of 2K bit and a real-time clock, both chips are mounted on the IIC bus.

EEPROM & RTC Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |  | description |
| --- | --- | --- | --- | --- |
| B4_B2_39 | 39 | LVCOMS33 | I2C data cable |  |
| B4_B3_42 | 42 | LVCOMS33 | I2C clock line |  |

## 10:25MHz crystal oscillator

The global clock of the EG4S20NG88 is provided by an active crystal with a frequency of 25MHz.

Clock Pin Assignment Table:

| Signal name | FPGA PIN | IOSTANDARD |  | description |
| --- | --- | --- | --- | --- |
| SYSCLK_25M | 34 | LVCOMS33 | Clock input |  |

## 11: CEP interface

MLKPAI-FMin01 leads most of the user IO of the EG4S20NG88, and if there is a signal that is multiplexed with the rest of the functions, please refer to the IO multiplexing table.

CEP Pin Assignment Table:

| J4 PIN |  | FPGA PIN | J5 PIN |  | FPGA PIN |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | \ | 1 | GND | \ |
| 2 | VCC_5V | \ | 2 | VCC_5V | \ |
| 3 | B4_BE2P_41 | 41 | 3 | B3_B5_30 | 30 |
| 4 | B4_BE2N_40 | 40 | 4 | B3_B6_31 | 31 |
| 5 | B4_BE1P_37 | 37 | 5 | B4_B1_35 | 35 |
| 6 | B4_BE1N_38 | 38 | 6 | B4_B2_39 | 39 |
| 7 | B3_BE2P_32 | 32 | 7 | B4_B3_42 | 42 |
| 8 | B3_BE2N_33 | 33 | 8 | B4_B4_44 | 44 |
| 9 | B1_L3P_12 | 12 | 9 | B5_R1_45 | 45 |
| 10 | B1_L3N_13 | 13 | 10 | B3_B1_23 | 23 |
| 11 | B3_BE1P_29 | 29 | 11 | B2_L2_19 | 19 |
| 12 | B3_BE1N_28 | 28 | 12 | B2_L1_18 | 18 |
| 13 | B1_L2P_10 | 10 | 13 | B1_L4_14 | 14 |
| 14 | B1_L2N_11 | 11 | 14 | B1_L2_3 | 3 |
| 15 | B7_TE1P_70 | 70 | 15 | B1_L1_2 | 2 |
| 16 | B7_TE1N_71 | 71 | 16 | B5_R2_51 | 51 |
| 17 | B8_TE1P_86 | 86 | 17 | B5_R3_52 | 52 |
| 18 | B8_TE1N_87 | 87 | 18 | B6_R1_57 | 57 |
| 19 | B6_R2P_64 | 64 | 19 | B6_R2_59 | 59 |
| 20 | B6_R2N_63 | 63 | 20 | B6_R3_62 | 62 |
| 21 | B1_L1P_5 | 5 | 21 | B6_R4_66 | 66 |
| 22 | B1_L1N_4 | 4 | 22 | B7_T4_72 | 72 |
| 23 | VCC_3V3 | \ | 23 | VCC_3V3 | \ |
| 24 | GND | \ | 24 | GND | \ |

# 5 EG4S20NG88 IO multiplexing table

The following is a table of IO resources for reuse:

| FPGA PIN |  | signal | function |
| --- | --- | --- | --- |
| 2 | B1_L1_2 |  | LED6、CEP J5 |
| 3 | B1_L2_3 |  | LED5、CEP J5 |
| 4 | B1_L1N_4 |  | DVP、CEP J4 |
| 5 | B1_L1P_5 |  | DVP、CEP J4 |
| 10 | B1_L2P_10 |  | DVP、CEP J4 |
| 11 | B1_L2N_11 |  | DVP、CEP J4 |
| 12 | B1_L3P_12 |  | DVP、CEP J4 |
| 13 | B1_L3N_13 |  | DVP、CEP J4 |
| 14 | B1_L4_14 |  | LED4、CEP J4 |
| 18 | B2_L1_18 |  | DIP switch, CEP J5 |
| 19 | B2_L2_19 |  | DIP switch, CEP J5 |
| 23 | B3_B1_23 |  | DIP switch, CEP J5 |
| 28 | B3_BE1N_28 |  | DVP、CEP J4 |
| 29 | B3_BE1P_29 |  | DVP、CEP J4 |
| 30 | B3_B5_30 |  | Buttons SW3, CEP J5 |
| 31 | B3_B6_31 |  | Buttons SW2, CEP J5 |
| 32 | B3_BE2P_32 |  | DVP、CEP J4 |
| 33 | B3_BE2N_33 |  | DVP、CEP J4 |
| 37 | B4_BE1P_37 |  | DVP、CEP J4 |
| 38 | B4_BE1N_38 |  | DVP、CEP J4 |
| 39 | B4_B2_39 |  | I2C、CEP J5 |
| 40 | B4_BE2N_40 |  | DVP、CEP J4 |
| 41 | B4_BE2P_41 |  | DVP、CEP J4 |
| 42 | B4_B3_42 |  | I2C、CEP J5 |
| 45 | B5_R1_45 |  | DIP switch, CEP J5 |
| 51 | B5_R2_51 |  | LED7、CEP J5 |
| 52 | B5_R3_52 |  | LED8、CEP J5 |
| 57 | B6_R1_57 |  | LED9、CEP J5 |
| 59 | B6_R2_59 |  | LED10、CEP J5 |

| 62 | B6_R3_62 | LED11、CEP J5 |
| --- | --- | --- |
| 63 | B6_R2N_63 | DVP、CEP J4 |
| 64 | B6_R2P_64 | DVP、CEP J4 |
| 70 | B7_TE1P_70 | DVP、CEP J4 |
| 71 | B7_TE1N_71 | DVP、CEP J4 |
| 86 | B8_TE1P_86 | DVP、CEP J4 |
| 87 | B8_TE1N_87 | DVP、CEP J4 |

# 6 Power Management

MLKPAI_FMin01 5V power supply is provided by TYPE-C, and the power-up timing of the power supply is as follows:

5

V

1.2

V

3.3

V

5

V

1.2

V

3.3

V

# 7 Naming conventions

Milianke hardware has a new naming convention, and for older models, the two names will be used at the same time

## 1: Core module naming convention

MLK – CXX - XX - XX

MILIANKEChip pin Num

Core

FamiliyChip feature Num

Num

## 2: Development platform naming rules

S - stands for Veneer

F-The development platform for FEP expansion interface in the form of core board + base plane

H - Development platform for FMC-HPC expansion interface in core board + backplane  mode L - Development platform for FMC-LPC expansion interface for core board + backplane

MLK - XX - CXX - XX

MILIANKE

S/F/H/L

NumCore Module